Follow your agency’s internal standard operating procedures.  If your agency does not have an internal standard operating procedure, the following standard operating procedure will be used for the region:

STEP 1 – IDENTIFY THE STATUS OF YOUR RADIO

- “SITE TRUNKING”—appears in radio display.

  - Coverage limited to your cell’s geographic coverage

  - Emergency buttons will work

- “FAILSOFT”—appears in radio display.

  - Radio will default to a single conventional channel

  - Coverage limited to your cells geographic coverage

  - Emergency button will not work

  - Single beep every 10 seconds

- “TOTAL SYSTEM FAILURE”

  - Display will show “OUT OF RANGE”

  - Radio will work in conventional mode (8M1 Zone).

  - Emergency button will not work

STEP 2 – TAKE ACTION BASED ON THE STATUS OF YOUR RADIO

- “SITE TRUNKING”

  - Monitor your radio for direction from your communications center

- “FAILSOFT”

  - Monitor your radio for direction from your communications center

  - Limit radio traffic to necessary communications

- “TOTAL SYSTEM FAILURE”

  - Activate the back-up communications plan for your agency as shown in the attached tables.

  - Monitor the assigned Dispatch/Command channel for direction

STEP 3 – IF UNABLE TO CONTACT YOUR COMMUNICATIONS CENTER

LAW ENFORCEMENT – Utilize “8TAC93” to communicate with San Diego Sheriff Communications.

Sheriff’s Communications can relay information to your communications center and give direction for what resources to switch to.

| LAW ENFORCEMENT BACK-UP RADIO RESOURCE FOR TOTAL HOME TRUNKED SYSTEM FAILURE |  |  |
| --- | --- | --- |
| Normal Talklgroups | System Back-up Dispatch & Command TG/Channel | System Back-up Tactical TG/Channel |
| City of San Diego System Failure |  |  |
| N/E DISP 1 N/W DISP 1 NOR DISP 1 EAS DISP 1 | RCS SD ICS1 | PRT RPT D |
| MID DISP 1 CEN DISP 1 | RCS SD ICS2 | PRT RPT D |
| WES DISP 1 S/E DISP 1 | RCS SD ICS3 | PRT RPT D |
| SD Community College PD – SDCC D1 SD Unified Schools PD – SCH PD | RCS SD ICS4 | PRT RPT D |
| RCS Failure |  |  |
| San Diego Sheriff VTAD (Vista Station) ENCD SMCD POWD | 8TAC91 Lake San Marcos Peak | CAR A-D |
| San Diego Sheriff VTAD (Fallbrook Station) | 8TAC94 Red Mountain | CAR A-D |
| San Diego Sheriff LGVD IMBD SNTD | CALAW8 Mt. San Miguel | CAR A-D |
| San Diego Sheriff RURD | LB East | CAR A-D |
| National City PD NATD Coronado PD CRDD Harbor PD HPD 1 & HPD2 | San Diego PD TAC 1 | CAR A-D |
| Chula Vista PD CHVD | San Diego PD TAC 4 | CAR A-D |
| Carlsbad PD CBDD Escondido PD ESCD | 8TAC92 Mt. Palomar | CAR A-D |
| El Cajon PD ELCD La Mesa PD LMSD | San Diego PD TAC 2 | CAR A-D |
| Oceanside PD OCSD | CITY 4 – City of Oceanside 800 MHz Back-up Repeater | CAR A-D |
| CHP El Cajon CHPELCSD & CHPELCED | CHP Low Band Gold 1 | CHP LB Channels |
| Imperial County Police Dispatch Talkgroups | CALAW8 Superstition Mountain; Cell phone and MDCs for departments without coverage | CAR A-D |
| SDSU PD SDSPD 1 & SDSPD2 | SDSU Police 800 MHz Repeaters | SDSU Simplex Channel |
| Cal State San Marcos PD CSU DISP 1 & CSU DISP 2 Palomar Comm College PD PALDISP Mira Costa Comm College PD MCCDISP | 8TAC91 Lake San Marcos with SDSO | CAR A-D |
| Grossmont/Cuyamaca College PD GRODISP1 & GRODISP2 UCSD PD UCSD PD D-1 & UCSD PD D-2 USD PD USDDISP | San Diego PD TAC 3 | CAR A-D |