Follow your agency’s internal standard operating procedures. If your agency does not have an internal standard operating procedure, the following standard operating procedure will be used for the region:

STEP 1 – IDENTIFY THE STATUS OF YOUR RADIO

- “SITE TRUNKING”—appears in radio display.

  - Coverage limited to your cell’s geographic coverage

  - Emergency buttons will work

- “FAILSOFT”—appears in radio display.

  - Radio will default to a single conventional channel

  - Coverage limited to your cells geographic coverage

  - Emergency button will not work

  - Single beep every 10 seconds

- “TOTAL SYSTEM FAILURE”

  - Display will show “OUT OF RANGE”

  - Radio will work in conventional mode (8M1 Zone).

  - Emergency button will not work

STEP 2 – TAKE ACTION BASED ON THE STATUS OF YOUR RADIO

- “SITE TRUNKING”

  - Monitor your radio for direction from your communications center

- “FAILSOFT”

  - Monitor your radio for direction from your communications center

  - Limit radio traffic to necessary communications

- “TOTAL SYSTEM FAILURE”

  - Activate the back-up communications plan for your agency as shown in the attached tables.

  - Monitor the assigned Dispatch/Command channel for direction

STEP 3 – IF UNABLE TO CONTACT YOUR COMMUNICATIONS CENTER

FIRE/EMS/LIFEGUARDS – Utilize “8CALL90” (in repeat mode) to communicate with San Diego Sheriff Communications.

Sheriff’s Communications can relay information to your communications center and give direction for what resources to switch to.

| OTHER SERVICES BACK-UP RADIO RESOURCE FOR TOTAL HOME TRUNKED SYSTEM FAILURE |  |
| --- | --- |
| Talklgroups | Back-up Radio Communications Resource |
| City of San Diego System Failure |  |
| Public Works and All Others | Cell phones |
| Life Guards | RCS COTAC4 |
| RCS Failure |  |
| All Others | CAR A,B,C,D and CAR5 and cell phones |
| Various City Life Guards on RCS | LGRN1 Conventional; Oceanside Lifeguards—CITY 4 for secondary back-up |